Lesson 33

Competencies:

10.1.5 Students will understand practical considerations of reactor design including materials of construction, mixing, heat transfer, and economics.

Economics- p22

CSTR/Batch Reactors (2004)

| Volume | Similar to: | Price |
| --- | --- | --- |
| 5 Gallons | Wastebasket | $29,000 |
| 50 Gallons | Garbage can | $38,000 |
| 500 Gallons | Jacuzzi | $70,000 |
| 1000 Gallons | 2 Jacuzzis | $85,000 |
| 4000 Gallons | Swimming Pool | $150,000 |
| 8000 Gallons | Gasoline Tanker | $280,000 |

What about heating and mixing issues?

Mixing

Not all CSTRs are ideally mixed.  This can lead to runaway reaction problems.

Mixing time video demonstration

NaOH reaction with HCl video

- Top tank- HCl

- Middle tank- NaOH

- Bottom tank- NaCl

Mixing versus reaction time:

Mixing time (95%) ≈ (20 to 50)/ N

Reaction time ≈ 1/k for 1st order

Space time () = VR/v0

CSTR well-mixed assumption valid if:

-  >> mix time

- 1/k >> mix time

Mixing versus reaction

Mixing Simulation

Probe locations PA, PB, PC, PD, PE

Scale-up

- As size increases, mixing takes longer.

- Mixing affects only fast reactions.

- Reaction may be masked by mixing time.

PSPB=NpρN3Di5SNpρN3Di5B=N3Di5SN3Di5B

S= scale-up    B= base case   NP= power number

N:  related to mixing time; tip speed

P:  energy required to stir

P/VR:  proportional to mass transfer (or heat)

Tip speed:  affects particles or cells (shear)

Summary

- Be careful of mixing time versus reaction time- kinetics can be masked

- You can have large concentration gradients depending upon stirrer

- Scale-up doesn’t keep all parameters the same- What is important?

- Stirrer design can make a difference

  - Silverson mixer video